ECHOLS COUNTY BOARD OF COMMISSIONERS

REQUEST FOR QUOTE

Replacement of 5-Ton Packaged HVAC System

Echols County Community Center – Church of God Street

Echols County Board of Commissioners is requesting written quotes from qualified HVAC contractors for the replacement of one existing 5-ton packaged heating and air-conditioning system at the Echols County Community Center located on Church of God Street in Statenville, Georgia.

SCOPE OF WORK

The successful contractor shall provide all labor, equipment, materials, tools, supervision, and incidentals necessary to complete the project, including:

- Remove and properly dispose of the existing HVAC unit.

- Furnish and install one new 5-ton packaged air-conditioning and heating unit.

- Provide 10 kW auxiliary electric heat.

- Furnish and install a new compatible digital thermostat.

- Remove and replace the existing rusted duct cover on the bottom of the unit/duct connection.

- Remove and replace the existing equipment pad with a new properly sized pad suitable for the new unit.

- Provide a complete, properly matched HVAC system.

- Reconnect electrical service, controls, condensate drainage, ductwork, and other existing building connections as necessary.

- Provide new disconnects, transitions, flashing, sealants, fasteners, and other installation materials where required.

- Properly seal all duct connections and penetrations to prevent air and water infiltration.

- Start, test, and verify proper operation of the new system, including cooling, heating, auxiliary heat, thermostat, and condensate drainage.

- Remove all equipment, debris, and materials associated with the work and leave the work area clean.

- Complete all work in accordance with applicable codes and manufacturer requirements.

SITE INSPECTION

Contractors are responsible for inspecting the existing equipment and site conditions before submitting a quote.

The contractor shall verify unit configuration, electrical requirements, duct dimensions and condition, equipment pad requirements, access, and all other conditions affecting installation.

The quote shall include all labor and materials necessary to provide the County with a complete and fully operational HVAC system.

QUOTE REQUIREMENTS

Each quote shall provide:

- Manufacturer and model of proposed equipment

- 5-ton cooling capacity

- Heating specifications

- 10 kW auxiliary heat

- SEER2/EER2 efficiency rating

- Equipment warranty

- Labor/installation warranty

- Total installed price

- Estimated equipment availability

- Estimated installation schedule

- Any exclusions or work not included in the quoted price

The total quoted price shall include all labor, materials, equipment, delivery, removal and disposal of existing equipment, new equipment pad, duct cover replacement, thermostat, permits if required, and all other costs necessary to complete the project.

CONTRACTOR REQUIREMENTS

The contractor shall be properly licensed and insured to perform HVAC work in the State of Georgia and shall provide evidence of licensing and insurance upon request.

QUOTE SUBMISSION

Quotes shall be submitted to:

Echols County Board of Commissioners
110 Highway 94 East
Statenville, Georgia 31648
Phone: 229-559-6538

Quote Deadline: Monday 17 2026, 4:30PM

Project Location:
Echols County Community Center
Church of God Street
Statenville, Georgia

Questions or requests to inspect the existing system may be directed to the Echols County Board of Commissioners at 229-559-6538.

Echols County reserves the right to reject any or all quotes, waive informalities, and select the quote determined to be in the best interest of the County. The County is not obligated to accept the lowest-priced quote.